Advanced Driver-Assistance Systems

SW252 - Course Project – Phase 2

Ahmed Wael 202201415

A. Component List

1. Arduino Uno Rev3 with CH340 Uploader

2. Breadboard 400 points

3. Bluetooth Module HC-05

4. L298 Motor Driver Module

5. 4 DC Geared Motor Dual Shaft 3:6VDC 450 rpm with Wheel (D: 65mm X W: 26mm)

6. HC-SR04 Ultrasonic Wave Detector Sensor 3.3V~5V

7. 4 Li-ion Battery 3.7V Rechargeable (Recycled) 18650-1200mAh

8. Dip Switch 8 Way

9. Photo-resistor LDR Light Sensor Module

10. Character LCD 2x16 Blue with Soldering Serial interface IIC/I2C Module

11. Red LED 5mm

12. Green LED 5mm

B. System architecture schematic

Connections Overview

1. Bluetooth Communication

- HC-05 TX → Arduino Pin 10

- HC-05 RX → Arduino Pin 11

- Used to receive:

  - Movement commands (F, B, L, R, S)

  - Speed levels (0–9, q)

- Used to send report for the connected device about:

  - Speed

  - Distance

  - Safety Status

2. Motor Control (L298 Module)

- ENA (PWM speed control) → Arduino Pin 3

- ENB (PWM speed control) → Arduino Pin 9

- IN1 → Arduino Pin 5

- IN2 → Arduino Pin 4

- IN3 → Arduino Pin 7

- IN4 → Arduino Pin 6

These pins control:

- Forward / backward movement

- Left / right turning

- Smooth acceleration and deceleration using PWM

3. Ultrasonic Sensor (HC-SR04)

- Trig → Arduino Pin 12

- Echo → Arduino Pin 8

Used to:

- Measure distance to obstacles

- Calculate Time-To-Collision (TTC)

- Reduce speed automatically when unsafe

4. Light Detection System

- LDR Output → Arduino A0

- Headlight LED → Arduino Pin 2

If ambient light is low, headlights turn ON automatically.

5. LCD Display (I2C)

- SDA → Arduino A4

- SCL → Arduino A5

Displays:

- Speed value

- Light status

- Obstacle safety state

6. Power Supply

- 4× Li-ion batteries

- Motor driver powered directly

- Arduino powered through regulated input

7. Safety Buttons Sensors

- Seat Belt Sensor → A1

- Door Lock Sensor → A2

Program Modules with Comments

1. Initialization Module (setup())

- Initialize serial communication

- Initialize Bluetooth

- Initialize LCD

- Configure all I/O pins

- Display startup message

2. Main Control Loop (loop())

- Read Bluetooth command (direction + speed)

- Read LDR for light level

- Read ultrasonic sensor for distance

- Apply incremental speed update

- Perform safety validation

- Execute movement (Forward / Reverse / Turn / Stop)

- Update LCD with system status

- Send Bluetooth telemetry logs

3. Bluetooth Command Processing

- Receives Bluetooth command characters

- Converts characters 0–9 and q into PWM speed values

- Stores target speed in Speed variable

- Does NOT move the car — only updates speed reference

- Send report to the connected device about (speed, light and safety state)

4. Ultrasonic Distance & Safety Module

- Measure obstacle distance

- Calculate Time To Collision (TTC)

- Reduce speed automatically when TTC is unsafe

5. Speed Control Module (Smooth Acceleration)

- Adjust motor speed

- Updates every 40 ms to ensure smooth motion

- Prevent sudden braking

- Uses PWM on ENA and ENB pins

6. Movement Control Module

| Command | Action |
| --- | --- |
| F | Move forward |
| B | Move backward |
| L | Turn left |
| R | Turn right |
| S / default | Stop |

Motor Direction Logic:
Implemented using IN1–IN4 pin combinations.

7. LDR

- Detect surrounding light

- Turn headlights ON/OFF automatically

8. LCD Display Module

Displays:

- Speed

- Light status (ON/OFF)

- Safety state (SAFE / OBSTACLE)

9. Safety Buttons Sensors

- Check the seatbelt and door before moving

- The car can move if the door is closed and the seatbelt is fastened

- Otherwise, the car will not move.

https://www.tinkercad.com/things/biWAMIYXoqj-phase-1-project?sharecode=WANjqpi-d3QQBnAoKnWevCtdt4a--Y42-4BOKmglXJU&authuser=0