Advanced Driver-Assistance Systems

SW252 - Course Project – Phase 1

Ahmed Wael 202201415

A. Component List

1. Arduino Uno Rev3 with CH340 Uploader

2. Bluetooth Module HC-05 3. L298 Motor Driver Module

4. DC Geared Motor Dual Shaft 3:6VDC 450 rpm with Wheel (D: 65mm X W: 26mm)

5. HC-SR04 Ultrasonic Wave Detector Sensor 3.3V~5V

6. 4 Li-ion Battery 3.7V Rechargeable (Recycled) 18650-1200mAh

7. Photo-resistor LDR Light Sensor Module

8. Character LCD 2x16 Blue with Soldering Serial interface IIC/I2C Module

9. Red LED 5mm

10. Green LED 5mm

B. System architecture schematic

Connections Overview

1. Bluetooth Communication

- HC-05 TX → Arduino Pin 10

- HC-05 RX → Arduino Pin 11

- Used to receive:

  - Movement commands (F, B, L, R, S)

  - Speed levels (0–9, q)

2. Motor Control (L298 Module)

- ENA (PWM speed control) → Arduino Pin 3

- ENB (PWM speed control) → Arduino Pin 9

- IN1 → Arduino Pin 5

- IN2 → Arduino Pin 4

- IN3 → Arduino Pin 7

- IN4 → Arduino Pin 6

These pins control:

- Forward / backward movement

- Left / right turning

- Smooth acceleration and deceleration using PWM

3. Ultrasonic Sensor (HC-SR04)

- Trig → Arduino Pin 12

- Echo → Arduino Pin 8

Used to:

- Measure distance to obstacles

- Calculate Time-To-Collision (TTC)

- Reduce speed automatically when unsafe

4. Light Detection System

- LDR Output → Arduino A0

- Headlight LED → Arduino Pin 2

If ambient light is low, headlights turn ON automatically.

5. LCD Display (I2C)

- SDA → Arduino A4

- SCL → Arduino A5

Displays:

- Speed value

- Light status

- Obstacle safety state

6. Power Supply

- 4× Li-ion batteries

- Motor driver powered directly

- Arduino powered through regulated input

C. Program Modules with Comments

1. Initialization Module (setup())

- Initialize serial communication

- Initialize Bluetooth

- Initialize LCD

- Configure all I/O pins

- Display startup message

2. Main Control Loop (loop())

- Continuously read inputs

- Apply safety logic

- Control motors

- Update display

3. Bluetooth Command Processing

- Maps Bluetooth characters (0–9, q) to PWM speed values

- Updates target motor speed

4. Ultrasonic Distance & Safety Module

- Measure obstacle distance

- Calculate Time To Collision (TTC)

- Reduce speed automatically when TTC is unsafe

5. Speed Control Module (Smooth Acceleration)

- Gradually adjust motor speed

- Prevent sudden acceleration or braking

- Uses PWM on ENA and ENB pins

6. Movement Control Module

| Command | Action |
| --- | --- |
| F | Move forward |
| B | Move backward |
| L | Turn left |
| R | Turn right |
| S / default | Stop |

Motor Direction Logic:
Implemented using IN1–IN4 pin combinations.

7. LDR

- Detect surrounding light

- Turn headlights ON/OFF automatically

8. LCD Display Module

Displays:

- Speed

- Light status (ON/OFF)

- Safety state (SAFE / OBSTACLE)

https://www.tinkercad.com/things/biWAMIYXoqj-phase-1-project?sharecode=WANjqpi-d3QQBnAoKnWevCtdt4a--Y42-4BOKmglXJU&authuser=0